| Commanding Officer USCGC ROLLIN FRITCH (WPC 1119) | 1 Munro Ave Cape May, NJ, 08204 |
| --- | --- |

FFsocial

FRITCHINST 5210.1A

11 March 2019

USCGC ROLLIN FRITCH (WPC 1119) INSTRUCTION 5210.1A

Subj:	GARBAGE MANAGEMENT PLAN

# Ref:	(a)	MARPOL Annex V, Regulations for the Prevention of Pollution by Garbage from Ships

# (b)	33 CFR 151.57, Waste Management Plans

# (c)	Shipboard Regulations Manual, COMDTINST M5000.7 (series)

# (d)	Vessel Environmental Manual COMDTINST M16455.1 (series)

# (e)	Coast Guard Food Service Manual, COMDTINST M4061.5 (series)

# (f)	Hazardous Waste Management Manual, COMDTINST M16478.1 (series)

# (g)	Classified Information Management Program, COMDTINST M5510.23 (series)

# (h)	Comptroller Manual, COMDTINST M4400.13 (series)

1.	PURPOSE.  This plan is written in accordance with the Vessel Environmental Manual, COMDTINST M16455.1 (series).  It promulgates necessary procedures and regulations to handle the garbage collection, retention, and disposal on board Coast Guard cutters in accordance with references (a) thru (h).

2.	ACTION.  Department Heads shall ensure compliance with this Instruction. All hands shall be familiar with the contents of this instruction.

3.	DIRECTIVES AFFECTED.  None.

4.	DISCUSSION.  References (a) and (b) require ships of 400 gross tonnage and over and certified to carry 15 persons or more (CG cutters 87 feet or larger in length) to carry a Garbage Management Plan and maintain a garbage record log.  In addition, every vessel of 26 feet or more in overall length (per 33 CFR 151.59 (a) (1)) shall display placards to notify the crew and passengers of disposal requirements.

5.	ENVIRONMENTAL ASPECT AND IMPACT.  None.

6.	FORMS AVAILABILITY.  None.

B. R. FIFER

Lieutenant, U.S. Coast Guard

Commanding Officer

Encl: 	(1) Garbage Management Plan

Garbage Management Plan

Chapter 1.  GENERAL GUIDELINES

- This Garbage Management Plan shall include a combination of three complementary techniques: (1) source reduction, (2) segregation/separation, and (3) disposal process.

- Source Reduction. When requisitioning supplies, only the required amount shall be taken on board to reduce generation of garbage.

- Segregation / Separation. Garbage is made up of distinct components. Each component should be evaluated separately to determine the best waste management practice for that waste.

- Disposal. Garbage disposal rules when in port and underway are different. Coast Guard cutters are generally equipped with one or more disposal devices (such as a trash compactor, incinerator, and/or macerator) for disposal at sea, depending on the class and endurance of the vessel. Garbage is always transferred to shore facilities when in port, in accordance with the host-tenant agreement.

Chapter 2. RESPONSIBILITIES

- Executive Officer

- Responsible for carrying out this Garbage Management Plan.

- Assigns locations of receptacles, collection, and separation stations.

- Determines training needed to facilitate collection, transportation, and disposal processes.

- Be familiar with overboard disposal restrictions and requirements of reference (d).

- Engineer Petty Officer

- Ensures all garbage handling equipment including, but not limited to, the compactor is in working order as per operation manual.

- Provides training to potential disposal equipment operators.

- Documents equipment operator qualifications if required.

- Provides the Master at Arms with adequate qualified personnel to operate the compactor.

- Be familiar with overboard disposal restrictions and requirements of reference (d).

- Master at Arms

- Ensures officers and crew are familiar with this instruction.

- Implements a compliance program to ensure everyone on board is being responsible, especially toward sorting of trash.

- Allocates garbage collection, separation, storage, and handling facilities.

- Ensures appropriate placards are positioned to notify all on board of disposal requirements of regulations 3 and 5 of reference (a).

- Ensures proper entries are made in the garbage record log.

- Identifies personnel responsible for the operation of garbage handling equipment and ensure they are qualified on the equipment assigned.

- Identifies available processing devices and their capacities. USCGC ROLLIN FRITCH (WPC 1119) is equipped with a model Whirlpool GX900QPPS4 compactor that has a compaction ratio of 4:1.

- Identifies how garbage will be transported between primary processing station and the storage or disposal stations.

- Identifies processing procedures considering (a) local recycling capabilities, (b) storage, and (c) disposal.

- Identifies the training or education to facilitate the processing of garbage.

- Determines how the garbage will be transported from the source of generation to the collection and separation stations.

- Determines how the garbage will be handled between primary collection and separation stations considering (a) possible local recycling, (b) on board processing, (c) storage, and (d) disposal at sea.

- Ensures the daily collection, separation, and transportation to allocated facilities when underway.

- Ensures the availability of allocated disposal facilities when in port.

- Supervises the disposal and/or destruction of collected trash while underway.

- Ensures that adequate supplies of paper and plastic trash bags are kept aboard.

- Food Services Officer

- Ensures food service members and mess attendants comply with this instruction.

- Supervises access to the trash handling area as needed by galley staff.

Chapter 3. PROCEDURES

- Provisioning

- To minimize waste generation, careful consideration should go into the types, quantities, and packaging of the organizational and personal provisions and parts brought on board.

- The use of disposable items should be avoided and used only when it is more prudent to do so (such as during AMIO or when in water conservation).

- Before getting under way, all unnecessary packaging from both organizational and personal items shall be disposed.

- For personal items, consider replacing or refilling near-empty containers prior to departure. Also, many items have inner and outer packaging, and it may be possible to discard outer packaging and leave only the inner while on board.

- In Port Collection (home port and away from home port)

- Collection begins at its source: (offices, workshops, berthing areas, galley, etc) and while in port shall be brought to the fantail receptacles at the end of the day.

- Clear bags shall be used whenever possible.

- In port refuse will be collected daily when “sweepers” is piped. Exceptions require approval from the Executive Officer or MAA.

- All refuse will be taken to the pier as directed by the Officer of the Day and the Plan of the Day.

- In port Disposal

- All refuse shall be disposed of in accordance with applicable local and state regulations.

- All garbage (and recyclables) will be taken directly to pier receptacles.

- The local U.S. Department of Agriculture Plant Protection and Quarantine representative will direct garbage disposal after arriving from any foreign country except Canada or offshore United States, per reference (c).

- Underway Collection

- The MAA shall oversee management and collection of trash in berthing and work place areas.

- Containers will be provided for the garbage and recyclables. All trash being collected shall be in clear plastic bags and is subject to inspection.

- Normally trash will be collected once per day underway at the conclusion of sweepers unless additional times are needed. Garbage will be taken directly to the fantail receptacles. The trash handling area will be maintained in an orderly and sanitary manner at all times.

- When permissible per Table 3-1 (on the following page), the MAA shall designate trash sorting parties to dispose of food wastes over the side. The MAA shall also manage a schedule of disposal of these items that is appropriate with the cutter’s routine of the day, in accordance with reference (a).

- Underway Retention

- All waste items will normally be retained on board per references (d), (e), and (f). If necessary the MAA shall take all steps to preserve the sanitation of the crew facilities by sorting, cleaning, and compacting items that cannot be disposed of at sea.

- The MAA shall make daily inspections of the trash handling area to ensure sanitary storage of all retained refuse.

- Underway Disposal

- All waste items will normally be retained on board as per references (d), (e), and (f). The MAA shall take all steps to preserve the sanitation of the crew facilities by sorting, cleaning, and compacting items that cannot be disposed of at sea.

- Classified Materials. No classified materials shall be stored in the trash handling area, disposed of to shore, or the sea unless properly destroyed first in accordance with reference (g).

| Garbage Type | Outside MARPOL Annex V Special Areas | Inside MARPOL Annex V Special Areas |
| --- | --- | --- |
| Plastics. This includes but is not limited to: food wrappings, Styrofoam, products for personal hygiene, packaging (vapor proof barriers, bottles, containers and liners), ship construction (fiberglass and laminated structures, siding, piping insulation, flooring, carpets, fabrics, adhesives, electrical and electrical components), disposable eating utensils and cups (including styrene products), bags, sheeting, floats, synthetic fishing nets, monofilament fishing line, strapping bands, hardhats, and synthetic ropes and lines. | Disposal Prohibited | Disposal Prohibited |
| Dunnage (material to prevent movement or damage of equipment or cargo), lining and packing materials | > 25 nm from nearest land | Disposal Prohibited |
| Paper, rags, glass, metal, crockery and similar refuse | > 12 nm from nearest land | Disposal Prohibited |
| All other garbage including paper, rags, glass, etc comminuted or ground. | > 3 nm from nearest land | Disposal Prohibited |
| Food waste not comminuted or unground | > 12 nm from nearest land | > 12 nm from nearest land |
| Food waste comminuted or ground | > 3 nm from nearest land | > 12 nm from nearest land |

Table 3-1

- Garbage Record Log

- While underway, and in port away from home port, each overboard discharge of garbage or incineration shall be recorded.

- Each record of discharge of garbage shall be signed for on the date of the discharge or incineration by the Officer of the Deck. Each completed page of the Garbage Record Log shall be signed by the Commanding Officer.

- The entry for each discharge or incineration shall include the date and time, position of the ship, description of the garbage and the estimated amount incinerated or discharged.

- The Garbage Record Log shall be kept on board the ship in Galley.

Chapter 4. OTHER REQUIREMENTS

- Personal Protective Equipment

- All persons involved in the collection, sorting, retention, processing, or disposal shall take prudent steps to protect themselves against disease or injury. At a minimum:

- Persons handling food or human refuse shall wear latex protective gloves to safeguard against blood borne pathogens and bacterial infections.

- Persons handling heavy materials shall wear safety boots.

- Persons disposing materials over the side of the ship shall don a type III or better Personal Flotation device. At night they shall have a spotter with a red flashlight.

- Placards

- One or more placards meeting the requirements of this section are displayed in prominent locations and in sufficient numbers so that they can be read by the crew and passengers. These locations must be readily accessible to the intended reader and may include embarkation points, food service facilities, garbage handling spaces, and common spaces on deck.

- Each placard must be at least nine inches wide by four inches high, made of a durable material, and lettered with letters at least 1⁄8 inch high.

- The placard must notify the reader of the following:

- The discharge of plastic or garbage mixed with plastic into any waters is prohibited.

- The discharge of all garbage is prohibited in the navigable waters of the United States and, in all other waters, within three nautical miles of the nearest land.

- The discharge of dunnage, lining, and packing materials that float is prohibited within 25 nautical miles of the nearest land.

- Other unground garbage may be discharged beyond 12 nautical miles from the nearest land.

- Other garbage ground to less than one inch may be discharged beyond three nautical miles of the nearest land.

- A person who violates the above requirements is liable for a civil penalty for each violation, and the criminal penalties of a class D felony.